实验报告

2019  年  4 月  2  日                                   成绩：

| 姓名 | 於文卓 | 学号 | 17061833 | 班级 | 17052317 |
| --- | --- | --- | --- | --- | --- |
| 专业 | 计算机科学与技术 |  | 课程名称 | 计算机组成原理课程设计 |  |
| 任课老师 | 冯建文 | 指导老师 | 冯建文 | 机位号 |  |
| 实验序号 | 2X | 实验名称 | 乘法器/除法器实验 |  |  |
| 实验时间 | 4月2日 | 实验地点 | 1-225 | 实验设备号 | #18 |
| 一、实验目的与要求 |  |  |  |  |  |
| - 实验目的： - 学习掌握如何配置IP核 - 学习使用IP核构建乘法器 - 掌握灵活运用Verilog HDL语言进行各种描述与建模的技巧和方法 - 实验要求： （1）使用IP核构建乘法器，要求输入大于等于8位 （2）通过板级验证 |  |  |  |  |  |
| 二、实验设计与程序代码 |  |  |  |  |  |
| - 模块设计说明 - 设计IP核，过程如下 - 在New Source页面选择创建IP核选项 - 在Math Function中选择乘法器 - 在配置页面选择位宽为8位无符号数 - 选择乘法器的构成（使用片上逻辑or内部乘法器），这里使用内部乘法器Mults - 定义输出的位宽 （2 ）构建顶层模块mul，并调用IP核 - 实验程序源代码及注释等 - IP核模块代码 - -- Interfaces: - -- a_intf - -- clk_intf - -- sclr_intf - -- ce_intf - -- b_intf - -- zero_detect_intf - -- p_intf - -- pcasc_intf - -- The following code must appear in the VHDL architecture header: - ------------- Begin Cut here for COMPONENT Declaration ------ COMP_TAG - COMPONENT multiply - PORT ( - clk : IN STD_LOGIC; - a : IN STD_LOGIC_VECTOR(7 DOWNTO 0); - b : IN STD_LOGIC_VECTOR(7 DOWNTO 0); - p : OUT STD_LOGIC_VECTOR(15 DOWNTO 0) - ); - END COMPONENT; - -- COMP_TAG_END ------ End COMPONENT Declaration ------------ - -- The following code must appear in the VHDL architecture - -- body. Substitute your own instance name and net names. - ------------- Begin Cut here for INSTANTIATION Template ----- INST_TAG - your_instance_name : multiply - PORT MAP ( - clk => clk, - a => a, - b => b, - p => p - ); - 顶层模块mul代码 - `timescale 1ns / 1ps - ////////////////////////////////////////////////////////////////////////////////// - // Company: - // Engineer: - // - // Create Date: 18:40:13 03/31/2019 - // Design Name: - // Module Name: mul - // Project Name: - // Target Devices: - // Tool versions: - // Description: - // - // Dependencies: - // - // Revision: - // Revision 0.01 - File Created - // Additional Comments: - // - ////////////////////////////////////////////////////////////////////////////////// - module mul( - input clk, - input [7:0] A, - input [7:0] B, - output [15:0]P - ); - multiply demo( - .clk(clk), - .a(A), - .b(B), - .p(P) - ); - endmodule |  |  |  |  |  |
| 三、实验仿真 |  |  |  |  |  |
| - 仿真代码 - `timescale 1ns / 1ps - //////////////////////////////////////////////////////////////////////////////// - module test; - // Inputs - reg clk; - reg [7:0] A; - reg [7:0] B; - // Outputs - wire [15:0] P; - // Instantiate the Unit Under Test (UUT) - mul uut ( - .clk(clk), - .A(A), - .B(B), - .P(P) - ); - initial - begin - clk = 0; - A = 0; - B = 0; - end - always #10 clk=~clk; - initial - begin - #20;A=1;B=1; - #20;A=1;B=2; - #20;A=1;B=20; - #20;A=1;B=64; - #20;A=3;B=3; - #20;A=4;B=4; - #20;A=5;B=5; - #20;A=6;B=6; - #20;A=10;B=10; - #20;A=14;B=12; - end - endmodule - 仿真波形 - 仿真结果分析 可以看到当A和B赋值后，clk经过一个上跳沿输入结果到P 当A为1时，P的值就为B的值 当A为3，B为3时，结果为9 当A为4，B为4时，结果为16 A为10，B为10，结果为100 A为14，B为12，结果为168 A为20，B为15，结果为300 仿真结果符合乘法法则，仿真结果正确 |  |  |  |  |  |
| 四、电路图 |  |  |  |  |  |
| 五、引脚配置（约束文件） |  |  |  |  |  |
| NET "clk" LOC = N17; NET "A[7]" LOC = V5; NET "A[6]" LOC = T4; NET "A[5]" LOC = V6; NET "A[4]" LOC = T5; NET "A[3]" LOC = T6; NET "A[2]" LOC = V7; NET "A[1]" LOC = R8; NET "A[0]" LOC = U9; NET "B[7]" LOC = V10; NET "B[6]" LOC = R10; NET "B[5]" LOC = U11; NET "B[4]" LOC = R11; NET "B[3]" LOC = U12; NET "B[2]" LOC = T13; NET "B[1]" LOC = V14; NET "B[0]" LOC = T14; NET "P[0]" LOC = V17; NET "P[1]" LOC = T16; NET "P[2]" LOC = V16; NET "P[3]" LOC = T15; NET "P[4]" LOC = U14; NET "P[5]" LOC = R13; NET "P[6]" LOC = U13; NET "P[7]" LOC = R12; NET "P[8]" LOC = V12; NET "P[9]" LOC = T11; NET "P[10]" LOC = V11; NET "P[11]" LOC = T10; NET "P[12]" LOC = V9; NET "P[13]" LOC = T8; NET "P[14]" LOC = U8; NET "P[15]" LOC = R7; NET "clk" IOSTANDARD = LVCMOS18 ¦CLOCK_DEDICATED_ROUTE = FALSE; NET "A[7]" IOSTANDARD = LVCMOS18; NET "A[6]" IOSTANDARD = LVCMOS18; NET "A[5]" IOSTANDARD = LVCMOS18; NET "A[4]" IOSTANDARD = LVCMOS18; NET "A[3]" IOSTANDARD = LVCMOS18; NET "A[2]" IOSTANDARD = LVCMOS18; NET "A[1]" IOSTANDARD = LVCMOS18; NET "A[0]" IOSTANDARD = LVCMOS18; NET "B[7]" IOSTANDARD = LVCMOS18; NET "B[6]" IOSTANDARD = LVCMOS18; NET "B[5]" IOSTANDARD = LVCMOS18; NET "B[4]" IOSTANDARD = LVCMOS18; NET "B[3]" IOSTANDARD = LVCMOS18; NET "B[2]" IOSTANDARD = LVCMOS18; NET "B[1]" IOSTANDARD = LVCMOS18; NET "B[0]" IOSTANDARD = LVCMOS18; NET "P[15]" IOSTANDARD = LVCMOS18; NET "P[14]" IOSTANDARD = LVCMOS18; NET "P[13]" IOSTANDARD = LVCMOS18; NET "P[12]" IOSTANDARD = LVCMOS18; NET "P[11]" IOSTANDARD = LVCMOS18; NET "P[10]" IOSTANDARD = LVCMOS18; NET "P[9]" IOSTANDARD = LVCMOS18; NET "P[8]" IOSTANDARD = LVCMOS18; NET "P[7]" IOSTANDARD = LVCMOS18; NET "P[6]" IOSTANDARD = LVCMOS18; NET "P[5]" IOSTANDARD = LVCMOS18; NET "P[4]" IOSTANDARD = LVCMOS18; NET "P[3]" IOSTANDARD = LVCMOS18; NET "P[2]" IOSTANDARD = LVCMOS18; NET "P[1]" IOSTANDARD = LVCMOS18; NET "P[0]" IOSTANDARD = LVCMOS18; //NET "SD1_" IOSTANDARD = LVCMOS18 ¦CLOCK_DEDICATED_ROUTE = FALSE;//下跳沿要用FALSE |  |  |  |  |  |
| 六、思考与探索 |  |  |  |  |  |
| - 实验结果记录： 在板级验证中 N17按钮控制时钟信号 开关V5到U9为A[7]到A[0] 开关V10到T14为B[7]到B[0] LED V17到R7共16位，其中V17表示P的低位 当T4和U12拨下，表示A=4 B=4 ，按下N17 U14LED灯亮起 表示P=16 当T6V7R8拨下 表示A=14 ，U12T13拨下表示B=12，此时T15R13R12灯亮起 表示P=168 若不拨下表示A的开关，拨下任何B的开关，灯均不亮，此时A=0 若不拨下表示B的开关，拨下任何A的开关，灯均不亮，此时B=0 - 实验结论： 实验结果也符合乘法法则，说明实验结果正确 - 问题与解决方案： - 不知道如何配置IP核 - 不知道在顶层模块中调用配置好的IP核 解决方案：通过网上搜索，查询了IP核的文档后，成功配置好了IP核， 使用这种形式调用IP核 - 思考题： |  |  |  |  |  |